|  |  | YES | N.A. | Comments |
| --- | --- | --- | --- | --- |
| 1 | SUBMITTALS |  |  |  |
| 1.1 | Architectural woodwork conforms to the submittals |  |  |  |
| 1.2 | Hardware conforms to submittals |  |  |  |
| 2 | CARE AND STORAGE |  |  |  |
| 2.1 | Temperature and humidity recorded (separate report attached to this document) |  |  |  |
| 3 | LUMBER |  |  |  |
| 3.1 | Correct cut (flat, quarter or rift) |  |  |  |
| 3.2 | Sound, no decay, shake, wane, pith, etc. |  |  |  |
| 3.3 | Natural characteristics: quantity, spacing & distribution |  |  |  |
| 3.4 | Conforms to minimum length, width & thickness for that specific species |  |  |  |
| 4 | SHEET PRODUCTS |  |  |  |
| 4.1 | Panel core (MDF, PB or ?) |  |  |  |
| 4.2 | Panel Balance (face/back) |  |  |  |
| 4.3 | HPDL Thickness |  |  |  |
| 4.4 | Veneer cut (rotary, flat, quarter or rift) |  |  |  |
| 4.5 | Veneer Match (within panel) |  |  |  |
| 4.6 | Veneer Grade |  |  |  |
| 4.7 | Solid surface, Phenolic, Other |  |  |  |
| 5 | FINISHING |  |  |  |
| 5.1 | System and specifications |  |  |  |
| 5.2 | Not Permitted or Inconspicuous defects: dust, runs, orange peel, sags, blistering, checking. etc. |  |  |  |
| 5.3 | Staining: consistency within panel & adjoining panels |  |  |  |
| 5.4 | Sheen as per specs. (do sheen test if necessary) |  |  |  |
| 5.5 | Back priming (millwork, wall surfacing, wood counters & premium grade casework) |  |  |  |

| 6 | MILLWORK | YES | N.A. | Comments |
| --- | --- | --- | --- | --- |
| 6.1 | Finger joints |  |  |  |
| 6.2 | Radius work (cord segmentation not permitted) |  |  |  |
| 6.3 | Door and window frames (thickness &type) |  |  |  |
| 6.4 | Wood panels, solid or veneer grade dependent |  |  |  |
| 6.5 | Shelf rods supported max. 48"(1219 mm) O.C. |  |  |  |
| 6.6 | Edge banding at shelve ends when required |  |  |  |
| 6.7 | Securely fastened |  |  |  |
| 6.8 | Installed plumb, level & square |  |  |  |
| 6.9 | Gaps, Flushness, Flatness & Alignment |  |  |  |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |
| A | Fabrication Gaps When Surfaces Are Mitered Or Butted |  |  |  |
| B | Fabrication Gaps When Parallel Pieces Are Joined |  |  |  |
| C | Fabrication Gaps When Edges Are Mitered Or Butted |  |  |  |
| D | Fabrication Flushness Between Two Surfaces |  |  |  |
| E | Flatness Of Panel Product |  |  |  |
| F | Overlap (Flushness Of Laminate) |  |  |  |
| G | Chip Out |  |  |  |
| H | Over Machining |  |  |  |
| I | Installation Gaps |  |  |  |
| J | Installation Flushness |  |  |  |

| 7 |  | STAIRWORK & RAILS |  | YES |  | N.A. |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1 |  | Delamination |  |  |  |  |  |  |  |
| 7.2 |  | Species, Grade & Defects |  |  |  |  |  |  |  |
| 7.3 |  | Treads, minimum 1” (25.4mm) |  |  |  |  |  |  |  |
| 7.4 |  | Closed stringers, minimum 3/4” (19mm) |  |  |  |  |  |  |  |
| 7.5 |  | Radius woodwork; Solid, Block laminated, Laminated Plies. Core veneered or Kerfed solid |  |  |  |  |  |  |  |
| 7.6 |  | Handrails/Guardrails fastenings |  |  |  |  |  |  |  |
| 7.7 |  | Compatible or Well matched for grain and colour |  |  |  |  |  |  |  |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |  |  |  |  |  |
| A |  | Fabrication Gaps When Surfaces Are Mitered Or Butted |  |  |  |  |  |  |  |
| B |  | Fabrication Gaps When Parallel Pieces Are Joined |  |  |  |  |  |  |  |
| C |  | Fabrication Gaps When Edges Are Mitered Or Butted |  |  |  |  |  |  |  |
| D |  | Fabrication Flushness Between Two Surfaces |  |  |  |  |  |  |  |
| E |  | Flatness Of Panel Product |  |  |  |  |  |  |  |
| F |  | Overlap (Flushness Of Laminate) |  |  |  |  |  |  |  |
| G |  | Chip Out |  |  |  |  |  |  |  |
| H |  | Over Machining |  |  |  |  |  |  |  |
| I |  | Installation Gaps |  |  |  |  |  |  |  |
| J |  | Installation Flushness |  |  |  |  |  |  |  |

| 8 | WALL/CEILING SURFACING & PARTITIONS | YES | N/A | Comments |
| --- | --- | --- | --- | --- |
| 8.1 | Core; material & thickness |  |  |  |
| 8.2 | Edge adhesive frees |  |  |  |
| 8.3 | Splines & Reveals; compatible/well matched |  |  |  |
| 8.4 | Splines & Reveals; compatible/well matched |  |  |  |
| 8.4 | Solid wood grade and cut per AWMAC’s STANDARDS or contract specifications |  |  |  |
| 8.5 | Veneer grade, cut and match per AWMAC’s STANDARDS or contract specifications |  |  |  |
| 8.6 | Veneer sequence panel to panel |  |  |  |
| 8.7 | Backs sealed (primed) |  |  |  |
| 8.8 | Grain loss at joints |  |  |  |
| 8.9 | Flitch transition at changes in plane |  |  |  |
| 8.10 | Flitch transition compatible/well matched |  |  |  |
| 8.11 | Colour and finish consistent |  |  |  |
| 8.12 | Finishing per AWMAC’s STANDARDS Section 5 |  |  |  |
| 8.13 | HPDL face and back thickness |  |  |  |
| 8.14 | Solid Phenolic thickness |  |  |  |
| 8.15 | Plumb |  |  |  |

Continued next page

| 8 | WALL/CEILING SURFACING & PARITIONS continued | YES | N.A. | Comments |
| --- | --- | --- | --- | --- |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |
| A | Fabrication Gaps When Surfaces Are Mitered Or Butted |  |  |  |
| B | Fabrication Gaps When Parallel Pieces Are Joined |  |  |  |
| C | Fabrication Gaps When Edges Are Mitered Or Butted |  |  |  |
| D | Fabrication Flushness Between Two Surfaces |  |  |  |
| E | Flatness Of Panel Product |  |  |  |
| F | Overlap (Flushness Of Laminate) |  |  |  |
| G | Chip Out |  |  |  |
| H | Over Machining |  |  |  |
| I | Installation Gaps |  |  |  |
| J | Installation Flushness |  |  |  |
| K | Installation Reveal At Adjacent Panels |  |  |  |
| L | Installation Flushness At Adjacent Panels |  |  |  |
| M | Fabrication Gap At Laminate Butt Jointed On The Same Piece Of Core |  |  |  |
| N | Show-Through Or Telegraphing |  |  |  |

| 9 | DOORS | YES | N.A. | Comments |
| --- | --- | --- | --- | --- |
| 9.1 | Per grade specified |  |  |  |
| 9.2 | Hardware blocking |  |  |  |
| 9.3 | Veneer grade cut and match per specified |  |  |  |
| 9.4 | Veneer match for paired doors, and/or transom adjoin |  |  |  |
| 9.4 | Veneer match doors to adjoining paneling |  |  |  |
| 9.5 | Edging per grade and specifications |  |  |  |
| 9.6 | Colour and finish consistent |  |  |  |
| 9.7 | Finishing per AWMAC’s STANDARDS Section 5 |  |  |  |
| 9.9 | Veneer and solid grain compatible/well matched |  |  |  |
| 9.7 | All edges sealed (use mirror) |  |  |  |
| 9.10 | Doors constructed to either WDMA performance standards and/or AWMAC’s STAMDARDS |  |  |  |
| 9.11 | Trimmed doors to AWMAC’s STAMDARDS, ask if any were trimmed and check edges for variation of edge finishes/machining as an indicator |  |  |  |
| 9.12 | Door frame thickness meets AWMAC’s STAMDARDS |  |  |  |
| 9.13 | Measure passage door faces at jamb, <1.6mm beyond and <3.2mm behind |  |  |  |
| 9.14 | Passage doors are aligned in frame (gaps 1/8") |  |  |  |
| 9.15 | Passage door, stile and rail at least one door, tests a, b, c, d |  |  |  |
| 9.15 | Door cutouts do not exceed maximum allowable area |  |  |  |
| 9.16 | Hardware installation (operates smoothly, neatly machined) e, m |  |  |  |

Continued next page

| 9 | Doors (continued) |  | YES |  | N/A |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |  |  |  |  |
| A |  | Fabrication Gaps When Surfaces Are Mitered Or Butted |  |  |  |  |  |  |
| B |  | Fabrication Gaps When Parallel Pieces Are Joined |  |  |  |  |  |  |
| C |  | Fabrication Gaps When Edges Are Mitered Or Butted |  |  |  |  |  |  |
| D |  | Fabrication Flushness Between Two Surfaces |  |  |  |  |  |  |
| E |  | Flatness Of Panel Product |  |  |  |  |  |  |
| F |  | Overlap (Flushness Of Laminate) |  |  |  |  |  |  |
| G |  | Chip Out |  |  |  |  |  |  |
| H |  | Over Machining |  |  |  |  |  |  |
| I |  | Installation Gaps |  |  |  |  |  |  |
| J |  | Squareness |  |  |  |  |  |  |
| N |  | Show-Through Or Telegraphing |  |  |  |  |  |  |

| 10 | CASEWORK | YES | N/A | Comments |
| --- | --- | --- | --- | --- |
| 10.1 | Interiors and exteriors of cabinets clean (e.g. edge tape glue) |  |  |  |
| 10.2 | Materials thickness and type (ends, bottoms, shelves, back, edge banding, doors and drawer fronts) Composite core? |  |  |  |
| 10.3 | Edges milled flush and eased |  |  |  |
| 10.3 | Wood veneer cuts (rotary, flat, quartered or rift) |  |  |  |
| 10.4 | Wood veneer matching and direction within panel |  |  |  |
| 10.5 | Veneer or pattern alignment & matching (sequencing) within a cabinet unit and/or vertically from cabinet to cabinet |  |  |  |
| 10.6 | Drawers slide freely |  |  |  |
| 10.7 | Drawer construction as per grade |  |  |  |
| 10.8 | Drawer box back and sub-front 5/8”(15mm) when greater than 30” (762mm) |  |  |  |
| 10.9 | Toe kick material |  |  |  |
| 10.10 | Fasteners at exposed interiors, are concealed or covered with colour matched caps |  |  |  |
| 10.11 | Remove at least one cabinet fastener of each type and check for size and type |  |  |  |
| 10.12 | Correct spacing of cabinet fasteners to wall and to each other |  |  |  |
| 10.13 | Scribing (caulking not exceed 1/8”(3.2mm) C&P |  |  |  |
| 10.14 | Closure panels installed |  |  |  |
| 10.15 | Maximum vertical cabinet door size 24’ x 84” (610mm x 2134mm) |  |  |  |
| 10.15 | Maximum horizontal cabinet door size 48” x 24’ (1219mm x 610mm) |  |  |  |
| 10.16 | Balanced construction of door and drawer fronts |  |  |  |
| 10.17 | Setback of fixed horizontal cabinet members |  |  |  |
| 10.18 | Cabinet shelving thickness correct for span and material |  |  |  |
| 10.19 | Adjustable shelving depth and end gaps |  |  |  |
| 10.20 | Hardware installation (operates smoothly, neatly machined) |  |  |  |
| 10.21 | Cabinet hinges securely fastened as per (screws, dowels) |  |  |  |
| 10.22 | Premium casework primed/sealed with 1 mil |  |  |  |
| 10.23 | Finishing |  |  |  |

Continued next page

| 10 |  | CASEWORK (continued) | YES | N.A. | Comments |
| --- | --- | --- | --- | --- | --- |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |  |
| A | Gaps When Surfaces Are Mitered Or Butted |  |  |  |  |
| B | Gaps When Parallel Pieces Are Joined |  |  |  |  |
| C | Gaps When Edges Are Mitered Or Butted |  |  |  |  |
| D | Flushness Between Two Surfaces |  |  |  |  |
| E | Flatness Of Panel Product |  |  |  |  |
| F | Overlap (Flushness Of Laminate) |  |  |  |  |
| G | Chip Out |  |  |  |  |
| H | Over Machining |  |  |  |  |
| I | Installation Gaps |  |  |  |  |
| J | Installation Flushness |  |  |  |  |
| N | Alignment of doors and drawer fronts |  |  |  |  |

f

| 11 | COUNTERTOPS | YES | N/A | Comments |
| --- | --- | --- | --- | --- |
| 11.1 | Scribing meets grade |  |  |  |
| 11.2 | Caulking applied consistently & not exceed 1/8”(3.2mm) C&P |  |  |  |
| 11.3 | Countertop, back splash and edge laminating sequence meets grade |  |  |  |
| 11.4 | Splash laminating meets grade |  |  |  |
| 11.5 | Backer sheets applied to all substrates |  |  |  |
| 11.5 | Sink countertop substrate meets grade |  |  |  |
| 11.6 | Counter joint distance to sink cutouts when discretionary (18” 457mm minimum) |  |  |  |
| 11.7 | Sink cutouts sealed |  |  |  |
| 11.8 | Installed within +/- 1/4" (6.4mm) of specified height |  |  |  |
| 11.9 | Top level within 1/8" in 96" |  |  |  |
| 11.10 | Horizontal reveal between lower edge of countertop and upper edge of door front meets grade |  |  |  |
| 11.11 | Measure unsupported countertop span greater than 1219mm, (48") |  |  |  |
| 11.12 | Cantilever less than maximum per standards (6" to 12") depending on material and thickness |  |  |  |
| 11.13 | Front and/or leading edge of countertop withstands upward pull of 75 lbs (34kg) |  |  |  |
| 11.14 | Cutouts radiused |  |  |  |
| 11.15 | Measure for consistent overhangs |  |  |  |
| 11.16 | Finishing |  |  |  |

Continued next page

| 11 |  | COUNTERTOPS (continued) | YES | N/A | Issues/Comments |
| --- | --- | --- | --- | --- | --- |
| Gaps, Flushness, Flatness & Alignment |  |  |  |  |  |
| A | Gaps When Surfaces Are Mitered Or Butted |  |  |  |  |
| B | Gaps When Parallel Pieces Are Joined |  |  |  |  |
| C | Gaps When Edges Are Mitered Or Butted |  |  |  |  |
| D | Flushness Between Two Surfaces |  |  |  |  |
| E | Flatness Of Panel Product |  |  |  |  |
| F | Overlap (Flushness Of Laminate) |  |  |  |  |
| G | Chip Out |  |  |  |  |
| H | Over Machining |  |  |  |  |
| I | Installation Gaps |  |  |  |  |
| J | Installation Flushness |  |  |  |  |
| N | Alignment of doors and drawer fronts |  |  |  |  |